SPARKER DC-CDI-P - mounting instructions for Husaberg FE650 with SEM ignition.

Wiring

Wiring must be done according the table below.

Note!!!

The wires DCCDIP unit for this bike has different colors than standard DCCDIP unit .

| wire color DC-CDI-P | wire color motorbike | name | description |
| --- | --- | --- | --- |
| orange |  | IC | ignition coil |
| green | orange | INPUT 1 | input 2 (used for kill switch) |
| black | grey | INPUT 2 | input 2 (used for 2nd curve) |
| yellow | green | CKPS | input for pick-up |
| blue | black | GND | ground |
| blue |  | GND | ground for ignition coil |
| violet |  | GEAR SHIFT LIGHT | output for gear shift light |
| red |  | + 12 V | supply 12 V (key switched) |
| green/yellow |  | TACHO | output for tachometer |

Producer address: 	IgniTech

Ceskobratrska 84

535 01 PRELOUC

Czech Republic

ignitech@ignitech.cz

www.ignitech.cz